MECH 151: Technical Design — Prototyping

Daily Formative Checks for CAD Understanding

Purpose: These are quick, ungraded checks-for-understanding meant to run inside the normal lecture/lab flow — not a fifth graded instrument alongside Attendance, the Design Portfolio, Unit Quizzes, and the Prototype. Each one targets whether students can actually do the day's CAD skill (not just recognize a term for it), so a misunderstanding surfaces the same day it's taught instead of at the Unit Quiz three weeks later.

How to run them: most take 3–5 minutes, at a natural break between lecture and hands-on lab time. Where an activity says "screenshot/hold up," a fast, low-effort signal (physical show-of-hands, colored card, or a one-line post to your LMS discussion board) is enough — the point is a same-day read on the room, not a collected artifact.

Dates follow the Fall 2026 schedule (Weeks 4–16). Sessions with no listed schedule topic (built-in lab/continuation time) get an activity that extends the prior session's skill; quiz days get a 60-second warm-up instead of a new activity so they don't eat into quiz time.

# Design Fundamentals (Foundations, before Unit 1)

Goal: Step back from the software to ask what a CAD model is for, before the sketching/modeling mechanics of Unit 1 begin.

Note: the weekly schedule table starts at Week 4 (your period of coverage); if Foundations content is covered before Week 4 or folded into early Unit 1 sessions, use this activity whenever that content actually lands.

| Session | Topic (from syllabus) | Quick Check (3–5 min) | What a Miss Looks Like |
| --- | --- | --- | --- |
| Foundations | Requirements vs. objectives vs. constraints; SMART goals | "Sort the card": each student writes one requirement, one objective, and one constraint for a made-up part on 3 sticky notes/cards. Swap with a neighbor, who sorts a stranger's 3 cards into R/O/C without being told which is which, then checks with the writer. | Neighbor can't tell a fixed limit (constraint) from an optimization target (objective) — e.g. calls "must weigh under 2 lb" an objective instead of a constraint. |

# Unit 1 — CAD Fundamentals (Basic): Weeks 4–7

Goal: Build the drawing and modeling foundation every later unit depends on.

| Session | Topic (from syllabus) | Quick Check (3–5 min) | What a Miss Looks Like |
| --- | --- | --- | --- |
| Wk 4 Wed Sep 16 | Sketching fundamentals; sketch constraints | "Turn it black": each student sketches an assigned simple profile (e.g. a slotted plate). Before the timer ends they must get every entity to fully-defined (black, not blue) and hold up a screenshot or show the sketch on their screen. | Sketch "looks right" but is still blue/under-defined, or the student added dimensions to lock it down without adding relations first (rigid but not robust to edits). |
| Wk 4 Thu Sep 17 | Orthographic projection; multiview theory | "Sketch the views, no CAD": display a simple 3D part (screen model or physical block) rotating in view. On paper/whiteboard, students freehand front/top/right views in 3 minutes, then compare with a neighbor. | Views aren't aligned to each other (top view not stacked directly above front view), or hidden lines for internal/back features are omitted entirely. |
| Wk 5 Wed Sep 23 | Descriptive geometry basics | "True length, live": put up a line that is oblique in 3D (angled in two view directions at once). Students individually state — before you demo it — which auxiliary construction would reveal its true length, then check against the demo. | Student tries to read true length directly off a principal view where the line appears foreshortened. |
| Wk 5 Thu Sep 24 | Primary and secondary views | "What view comes next?": show a primary auxiliary view already constructed for an inclined surface that itself has an oblique feature. Students identify — and sketch the projection arrow for — the secondary view still needed. | Student proposes projecting from one of the original six principal views instead of from the primary auxiliary view already on the board. |
| Wk 6 Wed Sep 30 | Basic dimensioning standards | "Dimension it, then audit it": hand out an un-dimensioned orthographic drawing. Students dimension it individually, then trade with a partner acting as "drafting inspector" who flags any violations (duplicate dimensions, dimensioning to a hidden line, missing a dimension) against a short checklist. | Dimensions placed on hidden (dashed) lines, the same size called out twice, or a dimension chain left open so a size can't be recovered without adding two others. |
| Wk 6 Thu Oct 1 | Introduction to parametric part modeling | "Predict, then rebuild": with a simple extrude+hole part on screen, announce a dimension change (e.g. "the hole diameter is now 12mm") before making it. Students write their one-line prediction of what will happen, then you rebuild and check. | Prediction only addresses the one feature changed and misses a downstream feature that references it (e.g. a fillet or pattern that depends on the hole). |
| Wk 7 Wed, Oct 7 — Unit 1 Quiz. Skip a new activity; instead open with one rapid-fire recall question (e.g. "how many DOF does one unconstrained circle sketch entity have?") as a warm-up, not for the activity slot. |  |  |  |

# Unit 2 — CAD Advanced: Weeks 7–10

Goal: Move from a basic model to a documentation-ready one that a shop can build from.

| Session | Topic (from syllabus) | Quick Check (3–5 min) | What a Miss Looks Like |
| --- | --- | --- | --- |
| Wk 7 Thu Oct 8 | Section views; auxiliary views; thread/fastener callouts | "Hatch or not?": rapid-fire show 5 features cut by a section plane (solid boss, rib, web, shaft, fastener). Whole room holds up Y/N cards for "gets hatched." Then: put "3/8-16 UNC-2B" on the board — each student decodes it alone on a card in 90 seconds and holds it up. | Rib/web/fastener gets hatched like solid material (the classic convention error); thread callout decode misses the class (fine vs. coarse) or the fit/class number. |
| Wk 8 Wed Oct 14 | Continued: detail drawings / section & auxiliary practice | "Peer drawing audit": trade in-progress detail drawings with a partner, who checks against a 4-item checklist (section labeled A-A style, auxiliary view has a fold/reference line, thread callout present, no orphaned dimension) and reports one fix. | Partner can't find the cutting-plane line on the drawing at all, or a thread callout is missing the class/fit designator. |
| Wk 8 Thu Oct 15 | Continued: detail drawings / section & auxiliary practice | "Swap and rebuild": students exchange model files (not drawings) with a neighbor and must generate one section view and one auxiliary view from someone else's part in the remaining lab time. | Student can operate the section/auxiliary tool but can't identify which cutting plane or reference plane is the correct choice for an unfamiliar part. |
| Wk 9 Wed Oct 21 | Production/detail drawings; title blocks; BOM basics | "Title block treasure hunt": pairs race to point out/label the title block, revision block, and BOM fields on a completed production drawing within 3 minutes. | Confuses the BOM with the title block, or can't locate where a revision would be recorded. |
| Wk 9 Thu Oct 22 | Introduction to tolerancing | "Fit or no fit?": present 3 shaft/hole tolerance pairs (e.g. ⌀25 H7/p6, ⌀25 H11/c11, ⌀25 H8/f7). Students compute max/min clearance or interference individually and classify each as clearance, interference, or transition. | Student calculates a number but can't say whether the parts will physically slide together or need to be pressed — i.e. gets the math but not what it means for assembly. |
| Wk 10 Wed Oct 28 | CAD software configuration; CAD/CAM data translation | "Format match": call out a use case ("send to a 3D printer," "archive for a customer with unknown CAD software," "keep editing the feature tree") — students answer STEP / STL / native with fingers (1–3) simultaneously. | Picks STL for a case needing an editable parametric model, or native format for exchange with an outside vendor on different software. |
| Wk 10 Thu, Oct 29 — Unit 2 Quiz; Prototype Groups Form. Skip the activity slot for group formation instead. |  |  |  |

# Unit 3 — Design for Assembly (DFA): Weeks 11–13

Goal: Combine part models into working, documented assemblies and apply DFA principles.

| Session | Topic (from syllabus) | Quick Check (3–5 min) | What a Miss Looks Like |
| --- | --- | --- | --- |
| Wk 11 Wed Nov 4 | Assembly modeling (mates/constraints); building assemblies; fasteners in assemblies | "Predict the DOF": live on screen, before confirming a new mate between two parts, students write down how many degrees of freedom will remain. Confirm the mate and check against the model. | Student can apply a mate that "looks right" on screen but can't say what motion is still free afterward. |
| Wk 11 Thu Nov 5 | Continued: assembly lab time | "Mate removal test": each student picks one mate in their own in-progress assembly, suppresses it, and predicts (then confirms with a partner) what motion becomes free again. | Removing the mate doesn't change behavior the student expected — usually a sign the mate was redundant with another one, or was never doing what they thought. |
| Wk 12 Wed Nov 11 | DFA principles (part count, ease of assembly, tolerance stack-up) | "Count and cut": show a 6-part bracket assembly. In pairs, propose one part-count reduction and justify the assembly-ease or cost benefit in 60 seconds; share out 2–3 pairs. | Proposed reduction breaks a stated requirement (e.g. removing a part that was providing required adjustability) — shows DFA is being applied without checking it against requirements. |
| Wk 12 Thu Nov 12 | Assembly drawings; exploded views; BOMs — Design Proposal due | "Balloon check": trade exploded-view drawings with a partner, who verifies every BOM line item has a matching balloon and vice versa. | A part appears in the model/BOM but has no balloon (or a balloon points at nothing) — the real-world QC failure this catches directly. |
| Wk 13 Wed Nov 18 | Design review / iteration workshop | "Two stars, one gap": structure the peer review students are already doing — each reviewer must name 2 specific things working and 1 specific requirement or objective not yet met, referencing the decision matrix from Foundations. | Feedback stays generic ("looks good," "needs work") instead of naming a specific requirement/objective — shows the decision-matrix habit from Foundations didn't carry forward. |
| Wk 13 Thu, Nov 19 — Unit 3 Quiz. Skip a new activity. |  |  |  |

# Unit 4 — Design for Manufacturing (DFM): Weeks 14–16

Goal: Close the loop from CAD model to physical part, including GD&T and manufacturing-process fit.

| Session | Topic (from syllabus) | Quick Check (3–5 min) | What a Miss Looks Like |
| --- | --- | --- | --- |
| Wk 14 Wed Nov 25 | Translating parametric models to manufacturing-ready files (GD&T introduced here if it lands this session) | "Export and inspect": students export one of their own curved features to STL and reopen/zoom the mesh to spot visible faceting versus the smooth parametric surface, then identify which export setting would reduce it. If GD&T is introduced this session, add: "decode one frame" — put up a single feature control frame; each student decodes the datum order and tolerance-zone type alone in 90 seconds before you reveal it. | Student exports without checking the mesh at all (treats export as a black box), or in the GD&T check, reads datums out of order (order sets priority in the datum reference frame). |
| Wk 14 Thu, Nov 26 — No class (Thanksgiving). |  |  |  |
| Wk 15 Wed Dec 2 | Manufacturing methods survey (additive/subtractive); DFM considerations; machine safety/PPE | "Which process?": show 3 part geometries (thin-wall shape, tight internal cavity, flat sheet with a hole pattern). Students vote 3D-print / CNC / laser and name one DFM constraint driving the choice. Before any hands-on machine use: rapid call-and-response or point-to-hazard walk on required PPE. | Picks a process without naming the geometric constraint that ruled out the alternatives (e.g. picks CNC for a part with an inaccessible internal cavity a cutter can't reach). |
| Wk 15 Thu Dec 3 | Prototype work time | "30-second team standup": each team states what's built, what's blocking them, and which requirement/objective they're furthest behind on. Spot-check one team's actual model file live against what they reported. | Team's verbal status doesn't match what's actually in their file (e.g. claims a joining method is done but the model shows two separate, unmated bodies). |
| Wk 16 Wed Dec 9 | Prototype work time | "Pre-flight checklist": teams self-check against a short printed list (drawings exported, GD&T callouts present, 2 bulk material types confirmed, joining method demonstrated) and initial each item before an instructor spot-check. | Team initials an item that isn't actually true yet — the spot-check is what catches this before the Final Exam, not after. |
| Wk 16 Thu, Dec 10 — Unit 4 Quiz. Skip a new activity. |  |  |  |

Final Exam — Monday, December 14 (Week 17)

This is the summative Prototype presentation, not a daily-activity day — the Week 16 pre-flight checklist above is the last formative check before it.