MECH 191 — Unit Quizzes

Applied, scenario-based short-answer quiz — each question requires using a formula, chart, or diagram from the course reference materials to reach an answer

MECH 191 — Metallurgy

Leeward Community College · Math and Science Division · Mechatronics Program

Name: _______________________________________        Date: _______________        Unit: 1

Student Handout — write your answers in the space provided

Unit 1 — Materials Fundamentals

1. Using the Hall-Petch relationship σᵧ = σ₀ + k/√d, a steel has σ₀ = 70 MPa and k = 18 MPa·mm^0.5. Calculate the yield strength for a fine-grained sample (d = 0.005 mm) and a coarse-grained sample (d = 0.05 mm). Which grain size would you specify for a high-strength structural bracket, and why?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

2. A tensile test on a steel specimen (original gauge length 50 mm) produces this data: the stress-strain curve becomes non-linear at 350 MPa, reaches a maximum stress of 550 MPa, and the specimen's final gauge length after fracture is 61 mm. Identify the yield strength, the ultimate tensile strength, and the % elongation at fracture. Is this steel ductile or brittle, and what visual feature would you expect on the fracture surface?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

3. Referencing the packing factors of common crystal structures (SC ~52%, BCC ~68%, FCC ~74%, HCP ~74%) and their typical slip behavior, a coworker claims a BCC metal will cold-form into a complex shape just as easily as an FCC metal. Is your coworker correct? Justify your answer using the structures' packing and slip-system differences.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

4. A quality engineer sectioning a failed part finds the crack initiated at a subsurface void roughly 0.5 mm across. Referencing the defect categories (point, line, planar, volume), which category does this void belong to, and how does its effect on strength differ from a single dislocation (a line defect) or a grain boundary (a planar defect)?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

5. An engineer must choose between a covalently-bonded ceramic and a metallically-bonded alloy for a bracket that will be dropped repeatedly during installation. Using what a metallic bond's "electron sea" allows compared to a ceramic's rigid, directional covalent bonds, which material is the safer choice for surviving impact, and why?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________