MECH 191 — Unit Quizzes

Applied, scenario-based short-answer quiz — each question requires using a formula, chart, or diagram from the course reference materials to reach an answer

MECH 191 — Metallurgy

Leeward Community College · Math and Science Division · Mechatronics Program

Name: _______________________________________        Date: _______________        Unit: 4

Student Handout — write your answers in the space provided

Unit 4 — Manufacturing Processes

1. A part is turned with a feed of 0.3 mm/rev using a tool with a 0.8 mm nose radius. Using Ra ≈ f²/(8r), calculate the theoretical surface roughness in μm. The print requires Ra ≤ 3.2 μm. Is this parameter set acceptable, and if not, name two parameter changes that would fix it without changing the tool.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

2. A radiograph of a casting shows an irregular, jagged void concentrated near the last region of the casting to solidify, rather than smooth, evenly distributed spherical voids. Referencing the difference between shrinkage porosity and gas porosity, which defect is this, and what change to the gating/risering system would you recommend?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

3. A shop needs to produce 500 identical brackets with a complex internal pocket and tight tolerances, with minimal per-part setup time after the first unit. Comparing sand casting, CNC milling, and powder metallurgy for this job, which would you recommend, and what's the key limiting factor ruling out the other two?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

4. An engineer compares a forged crankshaft to one machined from solid billet, both from the same alloy and final geometry. Referencing what happens to grain flow during forging versus machining, explain why the forged part is generally stronger along its load path.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

5. A powder metallurgy iron part is sintered at roughly 1100-1150°C — about 70-90% of iron's melting point, not fully melting the powder. Referencing the three sintering stages (burn-off, neck formation, densification/grain growth), explain why sintering works below the melting point rather than melting the part.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________