MECH 191 — Unit Quizzes

Applied, scenario-based short-answer quiz — each question requires using a formula, chart, or diagram from the course reference materials to reach an answer

MECH 191 — Metallurgy

Leeward Community College · Math and Science Division · Mechatronics Program

Name: _______________________________________        Date: _______________        Unit: 5

Student Handout — write your answers in the space provided

Unit 5 — Joining, Failure & Corrosion

1. A welder uses 25 V, 180 A, and a travel speed of 300 mm/min. Using HI = (V×A×60)/(1000×travel speed), calculate the heat input in kJ/mm. If the joint's WPS specifies a maximum heat input of 1.5 kJ/mm to avoid excessive HAZ grain coarsening, is this parameter set acceptable?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

2. A welding drawing shows a fillet weld symbol with "6" written to the left of the symbol on the arrow side of the reference line, and a tail containing the note "GMAW." Referencing weld symbol anatomy (reference line, arrow/other side, tail), what does the "6" specify, which side of the joint does the weld go on, and what does the tail note tell the welder?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

3. Referencing the galvanic series (most active/anodic like magnesium down to most noble/cathodic like gold/platinum, with steel and aluminum in between), an aluminum plate is fastened with stainless steel bolts in a marine environment. Which metal corrodes preferentially, and would the corrosion be worse with a small stainless bolt in a large aluminum plate, or a large stainless plate with small aluminum rivets? Explain using the area effect.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

4. A structural component fails unexpectedly in service. Walk through the standard failure analysis methodology in order (visual inspection → NDT → mechanical/metallographic testing → root cause → report), and explain why skipping straight to metallographic sectioning without a visual inspection first risks losing evidence needed to determine the failure's origin.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

5. A buried pipeline requires cathodic protection to a target of -0.85 V vs. Cu/CuSO4 per NACE SP0169. Referencing sacrificial anode vs. impressed current systems, if the pipeline is very long (50+ km) and needs a large protective current, which system is generally more practical, and why might a sacrificial anode system struggle over that distance?

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________